C:\Documents and Settings\julia.powell\My Documents\IHO TSMAD\S100-0 main\IHO S-100 Main Oct 1 2007.doc © ISO/IEC 2007 – All rights reservedISO-IEC_ 63Complementary elementIntroductory element — Main elementÉlément introductif — Élément central — Élément complémentaireIntroductory element — Main element — Complementary elementE2007-10-2 ISO/IECISO/IEC 2007 ISO/IEC ISO/IEC _(E). 2Heading 2Heading 1 02 STD Version 2.1c20 4

S-158:123

Published by the

International Hydrographic Organization

4b quai Antoine 1er

Principauté de Monaco

Tel: (377) 93.10.81.00

Fax: (377) 93.10.81.40

info@iho.int

www.iho.int

Marine Radio Services Validation Checks

Edition 1.0.0 – January 2026

Aligned to S-123 Edition 2.0.0

© Copyright International Hydrographic Organization 2026

This work is copyright. Apart from any use permitted in accordance with the Berne Convention for the Protection of Literary and Artistic Works (1886), and except in the circumstances described below, no part may be translated, reproduced by any process, adapted, communicated or commercially exploited without prior written permission from the International Hydrographic Organization (IHO). Copyright in some of the material in this publication may be owned by another party and permission for the translation and/or reproduction of that material must be obtained from the owner.

This document or partial material from this document may be translated, reproduced or distributed for general information, on no more than a cost recovery basis. Copies may not be sold or distributed for profit or gain without prior written agreement of the IHO Secretariat and any other copyright holders.

In the event that this document or partial material from this document is reproduced, translated or distributed under the terms described above, the following statements are to be included:

“Material from IHO publication [reference to extract: Title, Edition] is reproduced with the permission of the IHO Secretariat (Permission No ……./…) acting for the International Hydrographic Organization (IHO), which does not accept responsibility for the correctness of the material as reproduced: in case of doubt, the IHO’s authentic text shall prevail. The incorporation of material sourced from IHO shall not be construed as constituting an endorsement by IHO of this product.”

“This [document/publication] is a translation of IHO [document/publication] [name]. The IHO has not checked this translation and therefore takes no responsibility for its accuracy. In case of doubt the source version of [name] in [language] should be consulted.” The IHO Logo or other identifiers shall not be used in any derived product without prior written permission from the IHO Secretariat.

Document History

Changes to this Specification are coordinated by the S-123 Task Group (S-123 TG) of the IHO Nautical Information Provision Working Group (NIPWG). New editions will be made available via the IHO web site. Maintenance of the Specification shall conform to IHO Resolution 2/2007 (as amended).

| Version Number | Date | Author/Editor | Purpose |
| --- | --- | --- | --- |
| 1.0.0 Draft 1 | 2026-01-20 | Shwu-Jing Chang | Initial draft for S-123 Validation Checks |

Summary of Substantive Changes in Edition 1.x.x

Bold references in the Clauses Affected column indicate the principal sections/clauses that are affected by the described change.

| Change Summary | Clauses Affected |
| --- | --- |
| (To be populated for editions following Edition 1.0.0) |  |

Contents	Page

1	Introduction	1

1.1	Scope	1

1.2	Conformance	1

1.3	References	1

1.3.1	Normative references	1

1.3.2	Informative references	1

1.4	Terms, definitions and abbreviations	2

1.4.1	Terms and definitions	2

1.4.2	Abbreviations	2

1.4.3	Symbols	2

1.5	Use of language	2

1.6	General description	3

1.7	Specification metadata and maintenance	3

1.7.1	Specification metadata	3

1.7.2	Specification maintenance	3

2	Check Structure	5

3	Check Syntax	5

4	Organisation	5

5	Other Applicable Checks	5

5.1	Generic S-100 checks	5

5.2	Interoperability checks	6

5.3	Check messages	6

6	Check Application Sequence (informative)	6

7	Check Classification	7

8	Geometry and Spatial Operators	7

9	Other Components of this Specification	7

Introduction

This document specifies a set of checks that producers of S-123 Marine Radio Services validation tools must implement in their validation software in order to ensure conformance of S-123 datasets and exchange sets with requirements specified in the S-123 Product Specification. The initial list of checks for S-123 was compiled by the S-123 Task Group.

The checks listed in this document are product-specific. They supplement but do not replace the generic S-100 validation checks applicable to all S-100 products which are defined in a separate IHO publication (S-158:100 – Universal Hydrographic Model Validation Checks).

Scope

This document, designated as “S-158:123” by the IHO, specifies validation checks for data products conforming to Edition 2.0.0 of IHO Product Specification S-123 (Marine Radio Services).

This document specifies product-specific validation checks for both datasets and exchange sets containing datasets conforming to the listed Product Specifications.

The checks specified in this document supplement the checks described in Edition 1.0.0 of S-158:100 (Universal Hydrographic Data Model Validation Checks). Both sets of validation checks, those described in S-158:100 as well as those defined in S-158:123, must be applied to test the validity of S-123 datasets and exchange sets. For datasets and exchange sets intended for use on ECDIS, applicable additional cross-product checks defined in S-158:98, must also be applied.

The validation checks described in S-158:100, S-158:123, and S-100:98 do not cover all aspects of data quality; for example, these validation checks do not test whether encoded values are accurate compared with “ground truth” reality or observed values. These checks only cover adherence to requirements in relevant specifications (S-123, S-100, or S-98) and consistency within datasets, exchange sets, or (for the S-98 checks) with other datasets or data products.

Conformance

This specification conforms to Edition 1.0.0 of IHO specification S-158 (Validation Checks – Introduction and Structure).

Edition 1.0.0 is an Implementation version in accordance with IHO TR 2/2007 and there may be revisions issued by the Working Group prior to the Operational Edition 2.0.0 being published.

The validation checks described herein are intended for use with Edition 2.0.0 of IHO Product Specification S-123 (Marine Radio Services).

References

Normative references

S-100	IHO Universal Hydrographic Data Model, Edition 5.2.0, June 2024

S-123	Marine Radio Services Product Specification, Edition 2.0.0. In preparation.

S-158	Validation Checks – Introduction and Structure, Edition 1.0.0, February 2025.

S-158:100	Universal Hydrographic Data Model Validation Checks, Edition 1.0.0, February 2025.

Informative references

ISO 19157:2013	Geographic information – Data Quality. As amended by Amendment 1, 2018

S-98	Data Product Interoperability in S-100 Navigation Systems, IHO Publication S-98, Edition 2.0.0, October 2025.

S-158:98	Data Product Interoperability Validation Checks. Edition 1.0.0, February 2025.

Terms, definitions and abbreviations

Terms and definitions

The terms and definitions listed in S-158 apply to this document. In addition, the following terms and definitions are used in this document or Annex A:

known (attribute)

The XML element representing the attribute is found in the dataset and has been populated with a value.

NOTE: Validity of the attribute is not implied even if it is known.

missing (attribute)

The attribute is either not present, or is present but not populated.

optional (attribute)

The Product Specification permits the XML element to be omitted altogether from the dataset.

populated (attribute)

The attribute has a non-null value. String or text attributes whose value is the 0-length (empty) string are not considered populated, though empty strings are still valid strings and may be valid values for the attribute.

precede

To go before in time; to happen or exist before; to be earlier than.

present

The XML element representing the attribute is found in the dataset; it may or may not be populated with a value.

unknown (attribute)

The XML element representing the attribute is found in the dataset but has not been populated with a value.

valid (attribute)

The value of the attribute conforms to its value domain.

Antonym: invalid

NOTE: The empty string (0-length string) is prima facie a valid value for attributes of type text or CharacterString, though there may be additional requirements on string length imposed in the Product Specification, DCEG or Feature Catalogue.

value domain

set of accepted values [ISO/TS 19103:2005; from S-100 Edition 5.2.0 Annex A]

EXAMPLE The numeric range [3, 28]; all integers; any ASCII character; an enumeration of all accepted values, for example (green, blue, white).

Abbreviations

This Specification uses the abbreviated terms defined in S-158.

Symbols

The symbols used in logical and spatial expressions are defined in S-158 clause 1.3.3 (Symbols).

Use of language

Within this document:

- “Must” indicates a mandatory requirement.

- “Should” indicates an optional requirement, that is the recommended process to be followed, but is not mandatory.

- “May” means “allowed to” or “could possibly”, and is not mandatory.

General description

S-158:123 is a specification describing validation checks for S-123 data products. There are no data products based directly on this edition of S-158:123 and therefore no general information applicable to data products conforming to it.

Specification metadata and maintenance

Specification metadata

This information uniquely identifies this Specification and provides information about its creation and maintenance.

Title: 	Marine Radio Services Validation Checks

Version: 	1.0.0

Date: 	January 2026

Language: 	English

Classification: 	Unclassified

Contact: 	International Hydrographic Organization.

4 quai Antoine 1er,

B.P.445 MC 98011 MONACO CEDEX

Telephone: +377 93 10 81 00

B.P. 445 Fax: + 377 93 10 81 40

Email: info@iho.int

Role: 	Owner

URL: 	https://registry.iho.int

Identifier: 	S-158:123

Maintenance: 	Changes to this Specification are coordinated by the IHO Nautical Information Provision Working Group (NIPWG) and made available via the IHO Publications website. Maintenance of the Specification must conform to IHO Technical Resolution 2/2007 (revised 2010). For reporting issues which need correction, use the contact information.

Specification maintenance

Introduction

Changes to S-158:123 will be released by the IHO as a New Edition, revision, or clarification.

The list of checks, which accompanies this document is considered part of this Specification and changes to it are considered changes to this Specification.

S-158:123 is not accompanied by separate artefacts such as an XML Schema, Feature or Portrayal Catalogue and therefore this clause does not address the question of changes to such derived artefacts.

Note that the version numbering for S-158:123 conforms to the principles specified in S-158 Edition 1.0.0, which do not mandate a 1/1 correspondence between version numbers for S-123 and S-158:123.

New Edition

New Editions of S-158:123 include at least one of the following changes:

- introduce a new validation check (of any classification);

- remove an existing validation check (of any classification);

- change the classification of a validation check, whether upgrade (such as Error to Critical) or downgrade (such as Error to Warning);

- extend a validation check to include new features, conditions, etc., in a way that requires validation software manufacturers to change their software.

New Editions are likely to require validation software manufacturers to change their software or invalidate datasets which passed validation according to the previous Edition of S-158:123.

All cumulative revisions and clarifications must be included with the release of approved New Editions.

Revision

Revisions are defined as substantive semantic changes to S-158:123. Typically, revisions will change S-158:123 to correct factual errors or introduce necessary changes that have become evident as a result of practical experience or changing circumstances. Revisions include corrections of misinterpretations of S-123, or extensions to checks that do not require changes to validation software.

A revision must not be classified as a clarification. All cumulative clarifications must be included with the release of approved revisions.

Clarification

Clarifications are changes to S-158:123 arising from non-substantive reasons.

Typically, clarifications for non-substantive reasons remove ambiguity; correct grammatical and spelling errors; amend or update cross references; revise check messages or clarify check descriptions without requiring manufacturers to change their software.

Version numbers

The associated version control numbering to identify changes (boldface n) to S-158:123 must be as follows:

New Editions denoted as n.0.0

Revisions denoted as n.n.0

Clarifications denoted as n.n.n

Check Structure

Check structure in S-158:123 includes the fields specified in S-158 plus the additional fields specified in Table 2-1.

Table 2-1 – Extensions to check structure

| Column Name (Tag) | Description |
| --- | --- |
| Apply To | An indicator of check applicability by type of dataset: See Table 2-2 for the codes and their meanings. |
| Remarks | Optional notes pertaining to the check. |

Table 2-2 - Indicators for "Apply To" column

| Code | Subject of check | Description |
| --- | --- | --- |
| B | Base | Apply check to new dataset, new edition, and post-update dataset (after updates have been applied to the base). |
| U | Update | Apply check to update datasets in isolation. |
| S | Post-update | Apply check only to a post-update dataset (i.e., subsequent to application of all available updates). |

Check Syntax

The check syntax conforms to the syntax and operators for product-specific checks described in S-158 clause 4.2.

Organisation

The list of validation checks for this edition of S-158:123 is available separately (see clause 9). The list of checks accompanies this specification and forms an integral part of it.

Other Applicable Checks

Generic S-100 checks

S-123 datasets and exchange sets must also be validated using the following subset of the generic S-100 validation checks defined in S-158:100 Collection A.

Table 5.1 - Applicability of generic Collection A S-100 checks

| Document reference in S-158:100 list | Checks | Apply to | Remarks |
| --- | --- | --- | --- |
| Part 1 | N/A | N/A | No direct implementation on datasets or exchange sets |
| Part 2 / 2a | N/A | N/A | No direct implementation on datasets or exchange sets |
| Part 4a | N/A | N/A | No direct implementation on datasets or exchange sets |
| Part 4b | N/A | N/A | No direct implementation on datasets or exchange sets |
| Part 5 / 5a | All Collection A checks for Parts 5 and 5a | Datasets | The S-158:100 checks in Collection A compare datasets to the Feature Catalogue |
| Part 6 | N/A | Datasets | No direct implementation on datasets or exchange sets |
| Part 7 | All Collection A checks | Datasets | Except checks applicable only to spatial types not used by S-123 (for example multipoint) |
| Part 8 | None | N/A | Part 8 does not apply to S-123 |
| Part 9 / 9a / 13 | None | N/A | (Checks for Parts 9 / 9a / 13 are not defined in S-158:100 Edition 1.0.0) |
| Part 10a | None | N/A | S-123 does not use the ISO 8211 format |
| Part 10b | All Collection A checks | Dataset | Generic validation checks for the S-100 GML format |
| Part 10c | None | N/A | S-123 does not use the S-100 HDF5 format |
| Part 11 | All Collection A checks | Dataset | Checks for dataset size and CRS validity |
| Part 15 | All collection A checks | Exchange catalogue Exchange set | Checks for validity of digital signatures, which are mandatory in S-100 Edition 5.2.0 |
| Part 17 | All checks in Collection A except those applying to elements not used in S-123 | Exchange catalogue Exchange set |  |

Interoperability checks

S-123 datasets and exchange sets intended for use on ECDIS must also pass the applicable interoperability checks from those listed in S-158:98.

Check messages

Application software is expected to identify the location in the file where the error was detected, and indicate the location in addition to the check message. The method used to identify and indicate the location will depend on the format and is left to the application software developer.

Check Application Sequence (informative)

The suggested check application sequence is described in Table 6-1. Check developers and testers are free to apply the checks in a different order.

Table 6-1 - Suggested application order of validation checks

| Order | Check Collection | Defined in | Apply to |
| --- | --- | --- | --- |
| 1 | S-100 generic checks for datasets | S-158:100 | Dataset, in isolation |
| 2 | Product-specific checks for datasets | S-158:123 | Dataset, in isolation |
| 3 | S-100 generic checks for exchange sets | S-158:100 | Exchange set |
| 4 | Product-specific checks for exchange sets | S-158:123 | Exchange set |
| 5 | Any applicable interoperability checks | S-158:98 | Dataset or exchange set |
| Note: “Exchange set” in the above includes the exchange catalogue (CATALOG.XML), signatures, and any dataset or support files contained in an exchange set. |  |  |  |

Check Classification

The check classification and minimum standard described in S-158 apply to the checks defined in this S-158:123 specification.

Table 7.1 - Classification of checks by severity (from S-158)

| Indicator | Type | Description |
| --- | --- | --- |
| C | Critical Error | An error which would make a dataset or exchange set unusable in the end-user system through not loading; or causing an end-user system to crash; or presenting data which is unsafe or unintelligible for the intended purpose of the product, for example navigation. |
| E | Error | An error which may degrade the quality of the dataset or exchange set through appearance or usability but which will not pose a significant danger when used to support the intended purpose of the product. |
| W | Warning | An error which may be a duplication or an inconsistency which will not noticeably degrade the usability of a dataset or exchange set in the end-user system. |

Geometry and Spatial Operators

S-123 products use vector geometry. Any spatial operations mentioned in checks pertaining to vector spatial primitives must conform to the operations for vector products described in S-158:100.

For all spatial operators a default tolerance should be applied in validation software as follows:

- For coordinates in decimal degrees – a tolerance of 10-7 degrees or the precision of the lowest-precision coordinate field, whichever is greater.

- For coordinates in metres (UTM or UPS) – a tolerance of 0.01 m or the precision of the lowest-precision coordinate field being compared, whichever is greater.

Other Components of this Specification

The other components of this Specification listed below are provided as separate documents or artefacts accompanying this document and form an integral part of this Specification.

- Spreadsheet of S-123 validation checks named S158_123_1_0_0_YYYYMMDD. The build date is the YYYYMMDD suffix in the file name, whereas the “1_0_0” component represents the edition, revision, and clarification number. The file with the same edition and revision number as this document but with the most recent build date must be used.